BlackTip Jetsports Seat Cover Installation

Tools Needed

- Staple gun (electric or pneumatic preferred)

- At least 1/4" stainless steel staples, 3/8” crown

- Needle nose pliers

- Flat screwdriver or staple remover

- Scissors or utility knife

Installation

- Remove the original seat cover if necessary. It is necessary to remove it if equipped with pull tabs. These are for the seats with a higher back rest (lumbar). If the original cover is removed, it is recommended to cover the foam with plastic to help waterproof the seat.

- If the foam is wet or damaged, repair or replace it before installation.

- Warm the seat cover before installation using sunlight, a warm room, or a hair dryer.

- Position the BlackTip seat cover on the seat and align the front and rear.

- If equipped with pull tabs, (plastic tabs for lumbar section) feed them through the seat base, where the original tabs were removed, and secure them before final stapling.

- Starting at the front center, install 4–5 staples into the material.

- Staple the front corners while maintaining even tension.

- Stretch the cover toward the rear of the seat and staple the rear center, then the rear corners.

- Return to the front wings, stretch evenly from side to side, and staple securely.

- Fold and staple excess material in the corners for a clean finish.

- Complete stapling around the perimeter, adding additional staples as needed for a secure installation.

- Reinstall any straps, handles, or hardware removed during installation. BlackTip seat covers do not include replacement straps.

Tips

- Take your time stretching the cover for a smooth, wrinkle-free fit.

- Stainless steel staples are recommended to prevent corrosion.

- If wrinkles appear, remove a few staples and re-stretch before securing.

Questions?
Watercraft Superstore • 727-499-9707
watercraftsuperstore.com